The University of Western Ontario

Department of Electrical and Computer Engineering

Mid-term Examination			VLSI and Microelectronics (ECE480a)

Calculator and one additional equation sheet allowed

8:00 – 8:50, Thursday, Oct. 10, 2002			Instructor:  Dr. W. Wang

- (2 marks) Design a layout for the circuit fragment as shown in Fig. 1 in the two stages:

a) draw a stick diagram

b) finish the complete layout in Fig. 2 (using the P and N transistor layouts already shown in Fig. 2, draw on this paper)

Figure 1.

(In the actual picture, the poly is not  connected)

Figure 2

- (2 Marks) Justify

  - Why there is no required diffusion-metal spacing?

  - Why there is 2λ overhang of poly at transistor gate?

  - What does the tub tie connect and what is the purpose of tub tie?

  - What is DRC?

3.  (2 Marks) Assuming that Vdd= 5 V, Vss = 0V, Vgs = 3.3 V, Vtn= 0.7 V, W/L = 5/2 and V ds = 2 V, compute the drain current through the n-type transistor.

- (4 Marks) A two-input NOR gate drives three inverters (both static complementary gates) at the same time.  (Power supply voltage:  Vdd= 5 V, Vss = 0V.   All transistors have W=6λ, L=2λ, 1λ=0.25 um.  The unit gate capacitance of one n transistor or one p transistor is 0.9fFum2.)

  - Draw the schematic diagram for this NOR gate and the three inverters.

  - Compute the load capacitance on the NOR gate (Hint:  the output of the NOR gate connects to 6 transistors)

  - Based on the results of (a) and (b), using RC model (minimum size W=3λ, L= 2λ transistor has the resistance: Rn = 3.9 kΩ, Rp=14 kΩ, while RL=0Ω, no body-effect), compute the transition time (both rise and fall) of this NOR gate as between 5% of Vdd and 95% of Vdd.  (Only consider the worst case (the slowest case)).

Equations:

Gate capacitance equation: 	Cg=Cgunit∗W∗L

Saturation function: 		Id=0.5k'WL(Vgs−Vt)2

Linear function :  	 	Id=k'WL[(Vgs−Vt)Vds−0.5V2ds]

kn'=73uAV2, kp'=21uAV2

RC function:            		Vout(t)=Vdde−tRC

Inverter 90% -10% fall time:  tf=−(Rn+RL)CLln(0.10.9)

Inverter 10% - 90% rise time:  tr=−(Rp+RL)CLln(0.10.9)