# The University of Western Ontario

# Faculty of Engineering Science

# Department of Electrical and Computer Engineering

# Examination Booklet

Family name:		Given name:			Student number:

Midterm Examination in Introduction to VLSI (ECE349b)

### February 21, 2004   1:00pm to 2:00pm    TH 3154

This is an one-hour, closed-book essay examination with a simple calculator and one crib sheet (one-side) allowed.  Pencil writing is not allowed.

- The examination booklet consists of 6 pages including this front cover sheet.

- An parameter and equation sheet (page 2) is attached. A blank page is also attached for your convenience (page 6).

- Show all your work on the attached pages.  Any extra sheets will not be part of this examination and will not be marked.  You may use both sides of each page.

- Candidate is responsible for checking the completeness of the examination booklet and for writing his name and student number where indicated.

- State clearly any and all assumptions you make in arriving at a solution to a problem. If there is a doubt regarding the text of any of the questions, assumptions are to be made and solution presented based on the assumptions.

- In evaluating the examination, considerable weight will be given to the clarity of the answer, and to the presentation.

University policy states that cheating, including plagiarism, on an examination is a scholastic offense. The commission of a scholastic offense is attended by academic penalties which might include expulsion from the program. If you are caught cheating, there will be no second warning.

MARKS    Question 1:        /3			Question 3:                   /5

Question 2:        /2

Total                                       /10

The following information may be needed:

Parameters:

N-diffusion bottomwall unit capacitance: 0.6fFμm2

N-diffusion sidewall unit capacitance: 0.2fFμm

P-diffusion bottomwall unit capacitance: 0.9fFμm2

P-diffusion sidewall unit capacitance: 0.3fFμm

Metal1 plate capacitance: 0.09fFμm2

Metal1 fringe capacitance: 0.04fFμm

Metal1 square resistance:  4 Ω

Vtn = 0.7 V

Vtp = -0.8 V

Equations:

Gate capacitance equation: 	Cg=Cgunit∗W∗L

Saturation function: 		Id=0.5k'WL(Vgs−Vt)2

Linear function :  	 	Id=k'WL[(Vgs−Vt)Vds−0.5V2ds]

kn'=73uAV2, kp'=21uAV2

1.	(3 Marks)

- What is metal migration?  What is skin effect?

- Give the fabrication process of a transistor in four steps.

(c )

(d) What does the tub tie connect and what is the purpose of tub tie?

2. (2 Marks)   Assuming 0.5um technology, Vdd= 5 V, Vss = 0V, W/L = 5/2,  compute the source-drain current in the following transistor (Fig.1) and show the direction of the current in Fig. 1.

D = 0V

F=1V	                G=0V

Fig. 1

3. (5 Marks)  For the transistor in Question 2 above (Fig. 1),

- Draw the stick diagram and explain the reason of the poly overhang.

- Draw the layout (top-view) and show the sizes of W and L in terms of micrometer on the layout.

- Assuming the signals D, F, G coming from metal1 layer, how many contacts are needed to establish the connection as Fig. 1 and what layers does each contact connect?

- How many possible different capacitances are involved in this transistor and its necessary contacts? Explain how to calculate each capacitance.